Lifetime Brands

Robbinsville, NJ

Project #128175

Director IT

Host Interface Specification 1.8

Information in this document is subject to change without notice. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, for any purpose, without the express written permission of Dematic Corp.

2014 Dematic Corp. All rights reserved
Dematic confidential information. Not for further distribution.

This document contains confidential information, trade secrets and/or know-how which is the property of Dematic Corp., and may not be disclosed to any third party without the written permission of Dematic Corp.

Product and company names herein may be the trademarks of their respective owners.

Dematic Corp. shall hereinafter be referred to as Dematic.

Dematic Corp.
507 Plymouth Avenue
Grand Rapids, MI 49505
616-913-6200
800-530-9153
www.dematic.us

Release Dates

| Release Date | Description |  |
| --- | --- | --- |
| 2014-March-28 | Initial Version for POC 1.0 | A.J. Downer |
| 2014-May-6 | Initial version for Project 128175 | A.J. Downer |
| 2014-May-15 | Updated after on-site review with Lifetime | A.J. Downer |
| 2014-May-19 | Updated to use JSON formatting | A.J. Downer |
| 2014-June-12 | Removed all spaces from field names | A.J. Downer |
| 2014-June-19 | Added Voiceable fields for locations and carts. Changed some fields to alpha that were incorrectly spec’d as numeric | A.J. Downer |
| 2014-July-8 | Changes to drop off process | A.J. Downer |
| 2014-July-29 | As-built | A.J. Downer |
| 2014-August-18 | Added Spanish | A.J. Downer |
| 2014-September-2 | Added operator id to DROPCARTREQUEST | A.J. Downer |
| 2015-August-6 | Added additional handshakes to ensure data integrity | A.J. Downer |

Contents

1	Introduction	6

1.1	Purpose	6

1.2	Related Documents	6

1.3	Definition of Terms	6

2	Communications Architecture	7

2.1	Communication Protocol	7

2.2	Requirements for Using Message Distributor	7

2.2.1	Message Format	7

2.2.2	ACK Message Format	10

2.2.3	Message Distributor Protocol	10

2.2.4	Message Packaging	11

2.2.5	General Message Format	11

2.2.6	Predefined Server Message Types	12

3	Director IT 7.0 Interface	15

3.1	General Description	15

3.2	Director IT 7.0 Download Messages	15

3.2.1	PICKDOWNLOAD	17

3.2.2	CANCELREQUEST	19

3.2.3	NEXTCARTRESPONSE	20

3.2.4	DROPCARTRESPONSE	22

3.2.5	DROPSCANRESPONSE	23

3.3	Director IT 7.0 Upload Messages	24

3.3.1	ITEMSTATUS	25

3.3.3	CARTONSTATUS	27

3.3.4	CANCELSTATUS	27

3.3.5	CARTSTATUS	28

3.3.6	OPERATORSTATUS	29

3.3.7	NEXTCARTREQUEST	29

3.3.8	DROPCARTREQUEST	30

3.3.9	DROPCARTSCAN	30

4	Signatures	31

# Introduction

## Purpose

This document provides a definition of the Host Interface Specification for Dematic’s Director IT Picking system.  This Host Interface allows a higher-level system to have knowledge of the picking activity, and allows the Host system to update information Picking uses during processing.  The content of the Host Interface Specification defines the messages (data) exchanged by the interface and the communications protocol used for their exchange.

## Related Documents

Software Functional Specification (current version)

## Definition of Terms

| Acronym | Definition |
| --- | --- |
| GB | Giga Byte; one billion bytes (or characters) |
| HostComm | Program to interface with Lifetime Brands Host computer |
| MB | Mega Byte; one million bytes (or characters) |
| ODBC | Open Database Connectivity |
| PC | Personal Computer |
| WCS | Warehouse Control System |

# Communications Architecture

## Communication Protocol

The Host WMS will download and upload data to and from the Director IT 7.0 server via MessageDistributor TCP/IP socket interface.

WMS will cube, prioritize, and wave orders. Once released, the resulting orders are downloaded to the voice system server. Concurrently, paper work (labels) for the orders will be pre-printed by the WMS. These are distributed to the various pick modules accordingly.

## Requirements for Using Message Distributor

The following sections describe the Message Distributor protocol to be used in the interface between the WMS and the Dematic Director IT 7.0 server.

Socket client processes (belonging to the WMS) should connect to the socket server on port 7328.  Connecting clients must monitor their connection status and reconnect when necessary.  The desired filter message must be resent every time a reconnection is performed.

### Message Format

The following protocol packet format is used for sockets.

| Field | Field# | Type | Size (Bytes) | Units | Value | Description |
| --- | --- | --- | --- | --- | --- | --- |
| STX |  | ASCII | 1 | string | ASCII(02) | Start of Message. |
| PL | 1 | ASCII | Variable | string |  | Publisher Location – Typically the IP or computer name |
| PT | 2 | ASCII | Variable | string |  | Publisher Type – Typically the application name |
| SL | 3 | ASCII | Variable | string |  | Subscriber Location – Typically the IP or the computer Name |
| ST | 4 | ASCII | Variable | string |  | Subscriber Type – Typically the application name |
| MT | 5 | ASCII | Variable | string |  | Message Type |
| SN | 6 | ASCII | Variable | string |  | Sequence Number – Used to sequence the messages and to uniquely identify the message on this socket |
| Data | 7 | ASCII | Variable | NA |  | Data or payload |
| ETX |  | ASCII | 1 | NA | ASCII (03) | End of message. |

Note 1: If a sequence number is included in the message from the sender, all APPL type messages must be acknowledged by the receiver using an ACK message.

Note 2: No checksums are required as the underlying protocol guarantees uncorrupted receipt of messages.

Note 3: Acknowledgements are sent for repeated messages. Application data is not reprocessed. If an application receives two messages back to back that are identical then it should not process the second message.  This second message is most likely a result of the application not sending the acknowledgement message soon enough thus forcing the sending application to resend the message.  The receiver of the duplicate message must send an ack for the second message but it should not process it.

Note 4: TCP/IP provides a data stream on which messages are received. Messages can only be delimited using the prefix and postfix characters defined above. A message is rejected and considered not received if its message prefix and postfix characters are not received.

Field Descriptions

| Field | Description | Contents |
| --- | --- | --- |
| STX | Start of Text | ASCII Character 02 |
| PL | Publisher Location. Identifies the computer that is publishing the message. | IP address or computer name |
| PT | Publisher Type. Identifies the application that is publishing the message. The application that is publishing the message is tied directly to a Source Port at the TCP level but this field should not be the port number, it should be the application name that resides at that port. Think of it as a friendly name for the port, the same as the computer name is a friendly name for an IP address | (e.g. HOSTCOMM) |
| SL | Subscriber Location. Identifies the computer that is the intended recipient of the message. | IP address or computer name |
| ST | Subscriber Type. Identifies the application that is subscribing to the message. This application is tied directly to a Source Port at the TCP level but this field should not be the port number, but preferably the application name that resides at that port. Think of it as a friendly name for the port, the same as the computer name is a friendly name for an IP address | (e.g. ORDERCONTROL) |
| MT | Message Type |  |
| SN | Sequence Number – Used to sequence the messages and to uniquely identify the message on this socket. The sequence number is unique only in one direction on the socket and is not necessarily unique per application. In order to be truly unique at any given time the sequence number must be combined with the PL and PT field |  |
| Data | Data or payload | Message data (as described in message formats below) |
| ETX | End of Text | ASCII Character 03 |

### ACK Message Format

The following protocol packet format is used for acknowledging messages using a sockets interface.

| Field | Field# | Type | Size (Bytes) | Units | Value | Description |
| --- | --- | --- | --- | --- | --- | --- |
| STX |  | ASCII | 1 | string | ASCII(02) | Start of Message. |
| PL | 1 | ASCII | Variable | string |  | Publisher Location – The SL from the original message |
| PT | 2 | ASCII | Variable | string |  | Publisher Type – The ST from the original message |
| SL | 3 | ASCII | Variable | string |  | Subscriber Location – The PL from the original message |
| ST | 4 | ASCII | Variable | string |  | Subscriber Type – The PT from the original message |
| MT | 5 | ASCII | Variable | string |  | Message Type – Original Message Type + “_ACK” |
| SN | 6 | ASCII | Variable | string |  | Sequence Number – Original SN + “_ACK” |
| Data | 7 | ASCII | Variable | NA |  | NA |
| ETX |  | ASCII | 1 | NA | ASCII (03) | End of message. |

### Message Distributor Protocol

All processes that need to communicate to other processes will connect to the Message Distributor to establish these communication paths. These processes will be thought of as client processes to the Message Distributor, as they will implement the client side of the BSD sockets communication.

Any process can send a message, and the Message Distributor will distribute that message to any/all of the other processes. The distribution criteria are based upon the filtering information that each process supplied to the Message Distributor upon connection.

A process can create as many connections to the Message Distributor as necessary. It is important to think of messages at the connection level, not the process level, as one process may have multiple connections to the Message Distributor. This might be necessary when a process needs a connection to exchange data at a fast rate, such as when a real-time query is needed with the host to identify the carton destination. The other connection may have many lower priority messages queued awaiting processing with the host.

### Message Packaging

The messages of this interface are preceded by a Message_Prefix and followed by a Message_Postfix. The primary motivation for this is that messages sent over TCP/IP links are not always atomic or singular (one message per received packet).

The Message_Prefix is identified as the Start_Of_Message, using the ASCII STX, 0x02.  The Message_Postfix is identified as the End_Of_Message, using ASCII ETX, 0x03.  Each message must contain both a Message_Prefix and Message_Postfix. This allows the receiving software to determine the beginning and end of each message.

### General Message Format

Messages consist of a set of message fields, each field separated from other fields by a delimiter character (typically a ‘caret’ (^) ASCII value 94 (0x5E), ‘^’). Each of the fields may have a maximum size, but are typically variable in length.  If desired, padding characters (space for ASCII text fields, or leading zeros for numeric fields) may be used in a field, but their use is discouraged.

Note:  The caret (^) is used to separate fields.  Please do not place a caret (^) at the end of the message.

The data in a text field may consist of multiple sub-fields, but these sub-fields must be delimited with a character other than the standard field separator character (e.g., use ‘:’, ‘;’, or ‘/’ as sub-field delimiters). Any subscripted field (array) should be interpreted as a set number of separate fields also separated by carets. The Message_Prefix and Message_Postfix are not considered fields, and are not separated from the rest of the message by carets.

Note that there is no checksum, CRC or any other data integrity check field defined in a message. The assumption is that if the message is received by the target system, the message is correct. It is assumed that the transport layer will never deliver a damaged or out-of-sequence packet.

All messages have the header and footer described in the next section. The application payload fits in the Message Body (MB) field. All fields are separated by a delimiter (typically a caret “^”).

### Predefined Server Message Types

This section defines the Message Distributor control and system monitoring messages that are not governed by the client’s subscription.  These messages are considered level 4 messages in the 7 layer network architecture.

#### The Filter / Subscription Message ($filter$)

The Message Distributor retains only the last filter message received per client connection.  For any given connection, each subsequent filter message will replace any previously communicated filters.  The system can, however, have multiple connections with independent filtering for each connection.

A default filter of “0” is assigned to all new connections. Using this default, no messages will be distributed to this client until the filter has been explicitly changed by the client.  With that in mind, the filter message should be the first message sent by each client after it has received an OSI level network connection notification.

##### Description

The $filter$ message is used to sets the client’s filter. When the server receives a new filter message the old filter is replaced with the new filter. The server does not ACK this message.

##### Source/Destination

Source:	Any Client

Destination:	Message Distributor

##### Message Format

| Field | Field | Type | Size (Bytes) | Units | Value | Description |
| --- | --- | --- | --- | --- | --- | --- |
| STX |  | ASCII | 1 | string | ASCII(02) | Start of Message. |
| PL | 1 | ASCII | Variable | string |  | Publisher Location. |
| PT | 2 | ASCII | Variable | string |  | Publisher Type. |
| SL | 3 | ASCII | Variable | string |  | Subscriber Location |
| ST | 4 | ASCII | Variable | string |  | Subscriber Type. |
| MT | 5 | ASCII | 8 | string | $filter$ | Message Type. |
| SN | 6 | ASCII | 0 | string |  | Empty. |
| Filter | 7 | ASCII | Variable | NA |  | Data or payload. |
| ETX |  | ASCII | 1 | NA | ASCII (03) | End of message. |

##### Recommended / Default Filters

Host/WMS Download Process:

<STX>LIFETIMEHOST^WMS^^MessageDistributor^$filter$^^ST,’WMS’ ,=<ETX>

#### The Heartbeat Message ($heartbeat$)

##### Description

The $heartbeat$ is sent from the Message Distributor to any idle socket. The socket idle time is 60 seconds.

##### Source/Destination

Source:	Message Distributor

Destination:	Idle Socket Clients

##### Message Format

| Field | Field | Type | Size (Bytes) | Units | Value | Description |
| --- | --- | --- | --- | --- | --- | --- |
| STX |  | ASCII | 1 | string | ASCII(02) | Start of Message. |
| PL | 1 | ASCII | Variable | string |  | Publisher Location. |
| PT | 2 | ASCII | 18 | string | MessageDistributor | Publisher Type. |
| SL | 3 | ASCII | Variable | string |  | Subscriber Location |
| ST | 4 | ASCII | Variable | string |  | Subscriber Type. |
| MT | 5 | ASCII | 11 | string | $heartbeat$ | Message Type. |
| SN | 6 | ASCII | 0 | string |  | Empty. |
| Data | 7 | ASCII | 0 | NA |  | Empty. |
| ETX |  | ASCII | 1 | NA | ASCII (03) | End of message. |

# Director IT 7.0 Interface

## General Description

The following sections describe the messages between the Lifetime Brands Host and the Director IT 7.0 system.

All message formats listed below represent the Data portion of a complete message as defined in section 2.2.1.

Messages will use JSON formatting in the Data portion where fieldnames and their corresponding value are provided in pairs with both the fieldname and value surrounded in double quotes. The fieldname and value are then separated by a colon.  Fields may be sent in any order within the curly braces. Optional fields may be omitted entirely.

Example {“FIELD1”:”VALUE1”, “FIELD2”:”VALUE2”}

In the message definitions that follow, note that when the format of a field is given to be a certain number of bytes, this is the maximum length of the field. There is no need to pad data with spaces to meet the format length specified.

It is recommended that a delay of 50-100ms occur between each download message sent to the Director IT 7.0 system.

## Director IT 7.0 Download Messages

Downloads for the Voice Areas are sent to Director IT 7.0 through the MessageDistributor interface.  The messages in this section will define messages that Customer Host downloads to Director IT 7.0.

A PickDownload message is sent for each item to be picked.

NOTE: It is expected that similar line items will be consolidated per carton. For example, instead of sending three line items each requesting 2 eaches of the same product for a given carton, Dematic expects that one line item will be sent with a request of 6 eaches. However, if the request is instead for 2 each for three different cartons, three line items would be sent.

A CancelRequest message is sent to cancel a Cart, Carton or Line Item.

A NextCartResponse message is sent in response to a NextCartRequest message.

A CartStatusResponse message is sent to response to a CartStatus message.

A DropCartResponse message is sent in response to a DropCartRequest message.

### PICKDOWNLOAD

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘PICKDOWNLOAD’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID | Cart Label used for induction |
| OrderType | 13 bytes, alpha-numeric | M | CARTPICK | Order type |
| Zone | 20 bytes, alpha-numeric | M | Variable | Zone for Cart |
| OrderID | 50 bytes, alpha-numeric | M | Carton Label | SubUCC / Carton ID |
| CartSeq | 10 bytes, numeric | M | Variable | Cart Position of Order |
| ItemID | 30 bytes, alpha-numeric | M | Variable | Item ID/SKU |
| InnerCode | 30 bytes, alpha-numeric | M | Variable | Inner Code ID |
| CaseCode | 30 bytes, alpha-numeric | M | Variable | Case Code ID |
| UPC | 15 bytes, numeric | O | Variable | Universal Product Code |
| LineID | 10 bytes, alpha-numeric | O | Variable | Pick ID / Work Ref |
| Description | 30 bytes, alpha-numeric | M | Variable | Item Description EX: ALAVERT 24HR QCK DISOLV 12CT Note: All special characters will be parsed out by the Host. |
| UOM | 10 bytes, alpha-numeric | O | Variable | UOM Code Not used currently |
| UOM_SP | 10 bytes, alpha-numeric | O | Variable | UOM Code Not used currently |
| Location | 30 bytes, alpha-numeric | M | Location formats vary slightly from location to location. AAA-BBB-SP AAA – Aisle BBB – Bay SP – Shelf Position | Display Location (for screens) |
| VoiceAisle | 30 bytes, alpha-numeric | M | Variable | Voiceable Aisle. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| VoiceBay | 30 bytes, alpha-numeric | M | Variable | Voiceable Bay. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| VoiceShelf | 30 bytes, alpha-numeric | M | Variable | Voiceable Shelf. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| VoiceAisle_SP | 30 bytes, alpha-numeric | M | Variable | Voiceable Aisle (Spanish). Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| VoiceBay_SP | 30 bytes, alpha-numeric | M | Variable | Voiceable Bay (Spanish). Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| VoiceShelf_SP | 30 bytes, alpha-numeric | M | Variable | Voiceable Shelf (Spanish). Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| LocationSequence | 20 bytes numeric | O | Sequence for location. May not be unique. Secondary sort will be Location. | Walk sequence indicator for the location EX: 309 |
| CheckDigit | 5 byte, numeric | M | Variable | Last five digits of UPC |
| RequiredQuantity | 7 bytes, numeric | M | 1-9999999 | Quantity to Pick/Grab. EX: 5 |
| SpecialHandling | 50 bytes, alpha-numeric | O | Variable | Voiceable description of a special handling directive. |
| SpecialHandling_SP | 50 bytes, alpha-numeric | O | Variable | Voiceable description of a special handling directive in Spanish. |
| VoiceItemId | 1024 bytes, alpha-numeric | O | Variable | Possible UPCs for product verification separated by pipe (¦) |

#### Example PICKDOWNLOAD

Optional fields were omitted in this example

PLValue^PTValue^SLValue^STValue^PICKDOWNLOAD^{“MT”:”PICKDOWNLOAD”, “CartID”:”12345”, “OrderType”:”CARTPICK”, “OrderID”:”1111”, “CartSeq”:”1”, “Item ID”:”1”, “InnerCode”:”1234”, “CaseCode”:”5678”, “UPC”:”123234345456”, “LineID”:”1”, “Description”:”Sample Item”, “Location”:”A1-23-4B”, “VoiceAisle“:”alpha 1”, “VoiceBay“:”2 3”, “VoiceShelf“:”4 bravo”, CheckDigit”:”45456”, “RequiredQuantity”:”5”}

### CANCELREQUEST

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘CANCELREQUEST’ |  |
| CancelType | 50 bytes, alpha | M | Variable | Type of Deletion CART CARTON LINEITEM |
| CancelID | 50 bytes, alpha | M | Cart Manifest ID, Carton ID or Line ID | ID to delete |

### NEXTCARTRESPONSE

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘NEXTCARTRESPONSE’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID or “NOCARTS” | Cart Label used for induction. NOCARTS if no carts are available in the zone. |
| VoiceCart | 50 bytes, alpha | M | Variable | Voiceable cart value in the appropriate language for the operator. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| BuildStagingLoc | 50 bytes, numeric | M | Build Staging Location | Current Location of the Cart |
| VoiceStagingLoc | 50 bytes, alpha | M | Variable | Voiceable staging location in the appropriate language for the operator. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| OperatorID | 50 bytes, alpha | M | OperatorID from NEXTCARTREQUEST | Operator ID of requesting user |

### CARTSTATUSRESPONSE

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘CARTSTATUSRESPONSE’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID | Cart ID from the CARTSTATUS message |
| Status | 4 bytes, alpha | M | Status | Only populated if the Type of the CARTSTATUS message = DIRECTSCAN, WORKREFUSAL or AUTOASSIGNED. Either ‘PASS’ or ‘FAIL’. If the status fails, the operator will not be allowed to induct the Cart/SubUCC |

### DROPCARTRESPONSE

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘DROPCARTRESPONSE’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID | Cart Label used for induction |
| DropLocation | 50 bytes, alpha | M | Drop Location | Drop Location for the Cart |
| VoiceDropLoc | 50 bytes, alpha | M | Variable | Voiceable drop location in the appropriate language for the operator. Upper case characters will be voiced phonetically. Lower case characters will be spoken as words. Spaces should be used to separate characters, words and numbers. |
| SpecialHandling | 50 bytes, alpha | O | Variable | If “Y”, forces operator to scan exact location. Any other value is ignored. |

### DROPSCANRESPONSE

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘DROPSCANRESPONSE’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID | Cart Label used for induction |
| DropLocation | 50 bytes, alpha | M | Drop Location | Drop Location for the Cart |
| Status | 50 bytes, alpha | M | Variable | Either “OK” or “ERROR”. ERROR will direct operator to see supervisor. |

## Director IT 7.0 Upload Messages

This section of the document provides the details for messages originating from the Pick system destined for the Customer Host.

In the message definitions that follow, note that when the format of a field is given to be a certain number of bytes, this is the maximum length of the field. Upload data will not be padded with spaces to meet the format length specified.

There are six types of upload messages and two request messages.

A PickDownloadAck is sent for each PickDownload successfully processed.

An ItemStatus is sent for each item pick completed or partially completed.

A CartStatus is sent for each cart completed as well as for completed cartons that were picked as part of the Hot Carton process, and short completed cartons dropped off at the hospital area.

A CartonStatus is sent for cartons removed from a cart.

A CancelStatus is sent as a reply to a CancelRequest

An OperatorStatus is sent for each login or logoff.

A NextCartRequest is sent to get the next cart in the queue to be picked. WMS will respond with a NextCartResponse.

A DropCartRequest is sent to get a drop off location for a completed cart. WMS will respond with a DropCartResponse.

### PICKDOWNLOADACK

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘PICKDOWNLOADACK’ |  |
| CartID | 50 bytes, alpha | M | Cart Manifest ID | Cart Label acknowledged |
| LineID | 10 bytes, alpha-numeric | O | Variable | Pick ID / Work Ref acknowledged |

### ITEMSTATUS

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘ITEMSTATUS’ |  |
| CartID | 50 bytes, alpha | M | Cart ID | Cart Label used for induction (if the carton is being picked as a Hot Carton request, send the SubUCC) |
| OrderID | 50 bytes, alpha-numeric | M | Carton Label | Order ID / SubUCC |
| LineID | 15 bytes, alpha-numeric | O | Variable | Line ID |
| Location | 12 bytes, alpha-numeric | M | BBAARRL BB – Building AA – Area RR – Row L - Level | Location value |
| RequiredQuantity | 7 bytes, numeric | M | 1-9999999 | Quantity to Pick/Grab. EX: 5 |
| ActualQuantity | 7 bytes, numeric | M | 0-9999999 | Quantity Picked. EX: 5 |
| Date | 8 bytes, numeric, fixed length | M | mm/dd/yyyy | Date the pick was completed (for example, May 6, 2007 equals 05/06/2007) |
| Time | 8 bytes, alpha-numeric, fixed length | M | HH:MM:SS | Time the pick was completed (for example, 2:45:53 PM equals 144553) |
| OperatorID | 20 bytes fixed, alpha-numeric | M | Variable | Associate ID of who executed the pick |
| OperatorDescription | 255 bytes fixed, alpha-numeric | M | Variable | Associate Name of who executed the pick (i.e. Brian Smith) |
| State | 14 bytes, alpha | M | Variable | This field represents the state of the line item. CHOPUP COMPLETED SHORT SKIPSLOT PARTIAL-DAMAGE PARTIAL-WRONGITEM PARTIAL-NOSTOCK REJECTED SHORTCOMPLETED |

### CARTONSTATUS

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘CARTONSTATUS’ |  |
| OrderID | 50 bytes, alpha-numeric | M | Carton Label | Carton ID / SubUCC |
| Status | 50 bytes, alpha-numeric | M | Variable | Status HOTCARTON REMOVED |

### CANCELSTATUS

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘CANCELSTATUS’ |  |
| CancelType | 50 bytes, alpha | M | ‘Cart’, ‘Carton’ or ‘LineItem’ | Type of deletion |
| DeleteID | 50 bytes, alpha | M | Cart Manifest ID, Carton ID or Line ID | ID to delete |
| State | 15 bytes, alpha | M | Variable | CANCELLED FAILED |

### CARTSTATUS

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘CARTSTATUS’ |  |
| CartID | 50 bytes, numeric | M | Cart Manifest ID | Cart Label used for induction or the SubUCC for Hot Cartons or Cartons dropped off at hospital. |
| Date | 8 bytes, numeric, fixed length | M | mm/dd/yyyy | Date of event (for example, May 6, 2007 equals 05/06/2007) |
| Time | 8 bytes, alpha-numeric, fixed length | M | HH:MM:SS | Time of event (for example, 2:45:53 PM equals 144553) |
| OperatorID | 50 bytes, alpha | M | OperatorID | Operator ID |
| Type | 50 bytes, alpha | M | Variable | Induction method: DIRECTSCAN WORKREFUSAL AUTOASSIGNED Suspended flag: SUSPENDED Completed flag: COMPLETED |
| DropOffLocation | 50 bytes, alpha-numeric | M | Drop off Location | Not used for induction Last confirmed pick location for suspended (unless no picked completed, in which case the first location would be sent) Drop off location confirmed by operator for completed. |
| LineCount | 8 bytes, numeric | M | Number of lines on the cart | Total number of lines in the Cart or SubUCC.named in CartID. Only populated if Type = DIRECTSCAN, WORKREFUSAL or AUTOASSIGNED. |

### OPERATORSTATUS

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘OPERATORSTATUS’ |  |
| OperatorID | 50 bytes, alpha | M | OperatorID | Operator ID |
| TalkmanSerialNumber | 20 bytes, numeric | M | Talkman ID | Serial number of Talkman |
| Date | 8 bytes, numeric, fixed length | M | mm/dd/yyyy | Date of event (for example, May 6, 2007 equals 05/06/2007) |
| Time | 8 bytes, alpha-numeric, fixed length | M | HH:MM:SS | Time of event (for example, 2:45:53 PM equals 144553) |
| Status | 50 bytes, alpha | M | Variable | Either LOGON or LOGOFF |
| Zone | 50 bytes, alpha | M | Variable | Zone |

### NEXTCARTREQUEST

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘NEXTCARTREQUEST’ |  |
| Zone | 50 bytes, alpha | M | Zone | Operator Zone |
| OperatorID | 50 bytes, alpha | M | OperatorID | Operator ID of requesting user |

### DROPCARTREQUEST

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘DROPCARTREQUEST’ |  |
| CartID | 50 bytes, numeric | M | Cart Manifest ID | Cart Label used for induction or SubUCC of Carton for a Hot Carton |
| LastPickLoc | 50 bytes, numeric | M | Last Pick Loc | Last Pick Location visited by operator |
| OperatorID | 50 bytes, alpha | M | OperatorID | Operator ID of requesting user |

### DROPCARTSCAN

| Field | ###### Format | ###### Optional, Mandatory, or Not Applicable | ###### Value | ###### Description |
| --- | --- | --- | --- | --- |
| MT | 20 bytes, alpha | M | Always ‘DROPCARTSCAN’ |  |
| CartID | 50 bytes, numeric | M | Cart Manifest ID | Cart Label used for induction or SubUCC of Carton for a Hot Carton |
| DropOffLocation | 50 bytes, alpha-numeric | M | Drop off Location | Drop off location confirmed by operator. |

# Signatures

Document: 		Host Interface Spec 1.7
Revision Date:		2014-08-18
Customer:		Lifetime Brands, Inc.
Location:  		Robbinsville, NJ

To avoid schedule impact, all parties must sign this specification in the space provided below, and return it no later than the date specified in the e-transmittal of this document.

| Lifetime Brands | Lifetime Brands Representative | Lifetime Brands Representative (optional) |
| --- | --- | --- |
| Name |  |  |
| Signature |  |  |
| Title |  |  |
| Date |  |  |

| Dematic Corp. | SAE Software Project Lead | Technical Lead |
| --- | --- | --- |
| Name |  |  |
| Signature |  |  |
| Title |  |  |
| Date |  |  |